Chapter 2

Analyzing Technical Goals and Tradeoffs

Typical technical goals include scalability, availability, network performance, security, manageability, usability, adaptability and affordability. Tradeoffs have to be made in association with the technical goals. For example, meeting strict requirements for performance can make it hard to meet a goal of affordability.

Scalability

Scalability refers to how much growth a network design must support. For many enterprise network design customers, scalability is the primary goal. Many large companies add users, applications, additional sites, and external network connections at rapid rate. The network design you propose to a customer should be able to adapt to increase in network usage and scope.

Planning for expansion

To understand the expansion plan, ask your client how much the network will expand in the next year, in the next two years, in the next five years. Consider that many companies don’t have a clear five year vision.

Use the following questions to analyze your customer’s short-term goals for expansion

- How many more sites will be added in the next year? The next 2 years?

- How extensive will the networks be at each new site?

- How many more users will access the corporate internetwork in the next year? The next 2 years?

- How many more servers will be added to the internetwork in the next year? The next 2 years?

Expanding access to Data

Networks are expanded or grown to give more ways of access to Enterprise data. The business goal of making more data available to users results in the following technical goals for scaling and upgrading corporate enterprise network.

- Connect separated departmental LANs into the corporate internetwork.

- Solve LAN/WAN bottleneck problems caused by large increases in internetwork traffic.

- Provide centralized servers that reside in a data center.

- Make mainframe data accessible to the enterprise IP network.

- Add new sites to support field offices and telecommuters.

- Add new sites and services to support secure communication with customers, suppliers, resellers, and other business partners.

Constraints on Scalability

When analyzing a customer’s scalability goals, it is important to keep in mind that there are impediments to scalability inherent in networking technologies. Selecting technologies that can meet a customer’s scalability goals is a complex process with significant ramifications if not done correctly. For example, selecting a flat network topology with Layer 2 switches can cause problems as the number of user’s scales, especially if the users’ applications or network protocols send numerous broadcast frames. (Switches forward broadcast frames to all connected segments.)

Availability

Availability refers to the amount of time a network is available to users and is often a critical goal for network design customers. Availability can be expressed as a percent uptime per year, month, week, day, or hour, compared to the total time in that period. For example, in a network that offers 24-hour, 7-days-a-week service, if the network is up 165 hours in the 168-hour week, availability is 98.21 percent. In general, availability means how much time the network is operational. Availability is linked to reliability but has a more specific meaning (percent uptime) than reliability. Reliability refers to a variety of issues, including accuracy, error rates, stability, and the amount of time between failures.

Disaster Recovery

Companies needs to plan how to sustain a business and technical operations after a natural disaster such as floods, fires, hurricanes, and earthquakes, they also need to specify a recovery plan for unnatural disasters, such as bombs, terrorist, attacks, riots, or hostage situations. One of the most important steps in disaster recovery planning is testing. Not only must the technology be tested, but employees must be drilled on the actions they should take in a disaster. If the people don’t survive, the technology won’t help much. Also, people should practice working with the network in the configuration it will likely have after a disaster when redundant servers or sites are in use.

Specifying availability requirements

You should encourage your customers to specify availability requirements with precision.  Consider the difference between an uptime of 99.70 percent and an uptime of 99.95 percent. An uptime of 99.70 percent means the network is down 30 minutes per week, which is not acceptable to many customers. An uptime of 99.95 percent means the network is down 5 minutes per week, which might be acceptable, depending on the type of business. Availability requirements should be specified with at least two digits following the decimal point. It is also important to specify a timeframe with percent uptime requirements. Go back to the example of 99.70 percent uptime, which equated to 30 minutes of downtime per week. A downtime of 30 minutes in the middle of a working day is probably not acceptable.  But a downtime of 30 minutes every Saturday evening for regularly scheduled maintenance might be fine.

The cost of Downtime

In general, a customer’s goal for availability is to keep mission-critical applications running smoothly, with little or no downtime. A method to help you, the network designer, and your customer understand availability requirements is to specify a cost of downtime. For each critical application, document how much money the company loses per hour of downtime. (For some applications, such as order processing, specifying money lost per minute might have more impact.) If network operations will be outsourced to a third party network management firm, explaining the cost of downtime can help the firm understands the criticality of applications to a business’s mission. Specifying the cost of downtime can also help clarify whether in-service upgrades or triple redundancy must be supported.

Network performance definitions

Many network design customers cannot quantify their performance goals beyond, “It has to work with no complaints from users.” If this is the case, you can make assumptions about throughput, response time, and so on. On the other hand, some customers have specific performance requirements, based on a service level that has been agreed upon with network users. The following list provides definitions for network performance goals that you can use when analyzing precise requirements:

- Capacity (bandwidth): The data-carrying capability of a circuit or network, usually measured in bits per second (bps)

- Utilization: The percent of total available capacity in use

- Optimum utilization: Maximum average utilization before the network is considered saturated

- Throughput: Quantity of error-free data successfully transferred between nodes per unit of time, usually seconds

- Offered load: Sum of all the data all network nodes have ready to send at a particular time

- Accuracy: The amount of useful traffic that is correctly transmitted, relative to total traffic

- Efficiency: An analysis of how much effort is required to produce a certain amount of data throughput

- Delay (latency): Time between a frame being ready for transmission from a node and delivery of the frame elsewhere in the network

- Delay variation: The amount of time average delay varies

- Response time: The amount of time between a request for some network service and  a response to the request

Optimum network utilization

Network utilization is a measurement of how much bandwidth is used during a specific time period. Utilization is commonly specified as a percentage of capacity. For example, a network-monitoring tool might state that network utilization on an Ethernet segment is 30 percent, meaning that 30 percent of the capacity is in use. Optimum average network utilization is about 70 percent. A 70 percent threshold for average utilization means that peaks in network traffic can probably be handled without obvious performance degradation. Most WANs have less capacity than LANs, so more care is needed in selecting WAN bandwidth that can cover actual and reasonable variations.

Throughput

Throughput is defined as the quantity of error-free data that is transmitted per unit of time.  Throughput is often defined for a specific connection or session, but in some cases the total throughput of a network is specified. Network novices consistently misuse the words throughput and bandwidth. Remember, bandwidth means capacity and is generally fixed. Throughput is an assessment of the amount of data that can be transmitted per unit of time.

Accuracy

The overall goal for accuracy is that the data received at the destination must be the same as the data sent by the source. Typical causes of data errors include power surges or spikes, impedance mismatch problems, poor physical connections, failing devices, and noise caused by electrical machinery. Sometimes software bugs can cause data errors also, although software problems are a less common cause of errors than physical layer problems. Frames that have an error must be retransmitted, which has a negative effect on throughput. In the case of IP networks, Transmission Control Protocol (TCP) provides retransmission of data.

Efficiency

Efficiency is a term borrowed from engineering and scientific fields. It is a measurement of how effective an operation is in comparison to the cost in effort, energy, time, or money. Efficiency specifies how much overhead is required to produce a required outcome. For example, you could measure the efficiency of a method for boiling water. Does most of the energy go to actually boiling the water or does a lot of the energy get wasted heating the electrical wiring, the pot the water is in, and the air around it? How much overhead is required to produce the desired outcome? Efficiency also provides a useful way to talk about network performance. For example, shared Ethernet is inefficient when the collision rate is high. (The amount of effort to successfully send a frame becomes considerable because so many frames experience collisions.) Network efficiency specifies how much overhead is required to send traffic, whether that overhead is caused by collisions, token passing, error reporting, rerouting, acknowledgments, large frame headers, a bad network design, and so on.

Delay and delay variation

Users of interactive applications expect minimal delay in receiving feedback from the network. Voice and video applications also require minimal delay. In addition, voice and video applications require a minimal variation in the amount of delay that packets experience. Variations in delay, called jitter, cause disruptions in voice quality and jumpiness in video streams.

Causes of delay

Any goals regarding delay must take into account fundamental physics. Despite science fiction stories that say differently, any signal experiences a propagation delay resulting from the finite speed of light, which is about 300,000 kilometers per second (186,000 miles per second). Network designers can also remember 1 nanosecond per foot. These values are for light traveling in a vacuum. A signal in a cable or optical fiber travels approximately two-thirds the speed of light in a vacuum. Delay is relevant for all data transmission technologies but especially for satellite links and long terrestrial cables. Geostationary satellites are in orbit above the earth at a height of about 36,000 kilometers, or 24,000 miles. This long distance leads to a propagation delay of about 270 milliseconds (ms) for an intercontinental satellite hop. In the case of terrestrial cable connections, propagation delay is about 1ms for every 200 kilometers (120 miles).

Some other reasons for delay are

- Serialization delay, the time to put digital data onto a transmission line, which depends on the data volume and the speed of the line. For example, to transmit a 1024-byte packet on a 1.544-Mbps T1 line takes about 5ms.

- Packet switching delay refers to the latency accrued when switches and routers forward data. The latency depends on the speed of the internal circuitry and CPU, and the switching architecture of the internetworking device. Latency also depends on the type of RAM that the device uses.

- Packet-switching delay can also include queuing delay. The average number of packets in a queue on a packet-switching device increases exponentially as utilization increases.

Response Time

Response time is the network performance goal that users care about most. Users don’t know about propagation delay and jitter. They don’t understand throughput in pps or in MBps. They aren’t concerned about BERs, although perhaps they should be! Users recognize the amount of time to receive a response from the network system. They also recognize small changes in the expected response time and become frustrated when the response time is long.

Users begin to get frustrated when response time is more than about 100 ms or 1/10th of a second. Beyond 100 ms, users notice they are waiting for the network to display a web page, echo a typed character, start downloading email, and so on. If the response happens within 100 ms, most users do not notice any delay. The 100-ms threshold is often used as a timer value for protocols that offer reliable transport of data. For example, many TCP implementations retransmit unacknowledged data after 100 ms by default.

Security

Security is a key technical goal, and security design is one of the most important aspects of enterprise network design. Increased threats from both inside and outside the enterprise network require the most up-to-date security rules and technologies. An overall goal that most companies have is that security problems should not disrupt the company’s ability to conduct business. Network design customers need assurances that a design offers protection against business data and other assets getting damaged or accessed inappropriately. Every company has trade secrets, business operations, and equipment to protect. The first task in security design is planning. Planning involves identifying network assets that must be protected, analyzing risks, and developing requirements.

Identifying Network Assets

The first step in security design is identifying the assets that must be protected, the value of the assets, and the expected cost associated with losing these assets if a security breach occurs. Network assets include hardware, software, applications, and data. Assets also include intellectual property, trade secrets, and a company’s reputation. also include intellectual property, trade secrets, and a company’s reputation.

Analyzing security risks

In addition to identifying assets, an important step in security planning is analyzing potential threats and gaining an understanding of their likelihood and business impact. Risk analysis and the consequent building of a security policy and secure network design is a continuous process, as risks change in their severity and probability on a regular basis. For example, a company’s encryption algorithm and the length of the encryption key might need to be reconsidered if a relatively inexpensive and exceptionally fast code cracking computer becomes available, which allows easier decryption of valuable secrets.

Developing Security Requirements

Security problems should not disrupt an organization’s capability to conduct business. That’s the most basic security requirement that every organization has. A secondary security requirement is to protect assets from being incapacitated, stolen, altered, or harmed. Although every design customer has different detailed security requirements, basic requirements boil down to the need to develop and select procedures and technologies that ensure the following:

- Confidentiality of data so that only authorized users can view sensitive information

- Integrity of data so that only authorized users can change sensitive information and so that authorized users of data can depend on its authenticity

- System and data availability, which should provide uninterrupted access to important  computing resources

Other, more specific requirements could include one or more of the following goals:

- Let outsiders (customers, vendors, suppliers) access data on public web or File Transfer Protocol (FTP) servers but not access internal data.

- Authorize and authenticate branch-office users, mobile users, and telecommuters.

- Detect intruders and isolate the amount of damage they do.

- Authenticate routing-table updates received from internal or external routers.

- Protect data transmitted to remote sites across a VPN.

- Physically secure hosts and internetworking devices (for example, keep devices in a locked room).

- Logically secure hosts and internetworking devices with user accounts and access rights for directories and files.

- Protect applications and data from software viruses.

- Train network users and network managers on security risks and how to avoid security problems.

- Implement copyright or other legal methods of protecting products and intellectual property.

- Meet compliance and regulatory requirements.

Manageability

Every customer has different objectives regarding the manageability of a network. Some customers have precise goals, such as a plan to use SNMP to record the number of bytes each router receives and sends. Other clients have less-specific goals. If your client has definite plans, be sure to document them, because you will need to refer to the plans when selecting equipment. In some cases, equipment has to be ruled out because it does not support the management functions a customer requires.

ISO uses the FCAPS acronym to help you remember the following network management functions:

- Fault management: Detecting, isolating, and correcting problems; reporting problems to end users and managers; tracking trends related to problems

- Configuration management: Controlling, operating, identifying, and collecting data from managed devices

- Accounting management: Accounting of network usage to allocate costs to network users and/or plan for changes in capacity requirements

- Performance management: Analyzing traffic and application behavior to optimize a network, meet service-level agreements, and plan for expansion

- Security management: Monitoring and testing security and protection policies, maintaining and distributing passwords and other authentication and authorization information, managing encryption keys, and auditing adherence to security policies

Usability

A goal that is related to manageability, but is not exactly the same as manageability, is usability. Usability refers to the ease of use with which network users can access the network and services. Whereas manageability focuses on making network managers’ jobs easier, usability focuses on making network users’ jobs easier. It is important to gain an understanding of how important usability is to your network design customer; because some network design components can have a negative effect on usability. For example, strict security policies can have a negative effect on usability (which is a tradeoff that most customers are willing to make, but not all customers). You can plan to maximize usability by deploying user-friendly, host-naming schemes and easy-to-use configuration methods that make use of dynamic protocols, such as the

Dynamic Host Configuration Protocol (DHCP).

Adaptability

When designing a network, you should try to avoid incorporating any elements that would make it hard to implement new technologies in the future. A good network design can adapt to new technologies and changes. Changes can come in the form of new protocols, new business practices, new fiscal goals, new legislation, and a myriad of other possibilities.

A flexible network design can also adapt to changing traffic patterns and QoS requirements. For some customers, the selected WAN or LAN technology must adapt to new users randomly joining the network to use applications that require a constant-bit-rate service. One other aspect of adaptability is how quickly internetworking devices must adapt to problems and to upgrades. For example, how quickly do switches and bridges adapt to another switch failing, causing a change in the spanning-tree topology? How quickly do routers adapt to new networks joining the topology? How quickly do routing protocols adapt to link failures?

Affordability

The final technical goal is affordability which is sometimes called cost-effectiveness. Most customers have a goal for affordability, although sometimes other goals such as performance and availability are more important. For a network design to be affordable, it should carry the maximum amount of traffic for a given financial cost. Financial costs include nonrecurring equipment costs and recurring network operation costs.

In campus networks, low cost is often a primary goal. Customers expect to be able to purchase affordable switches that have numerous ports and a low cost per port. They expect cabling costs to be minimal and service provider charges to be minimal or nonexistent. They also expect NICs for end systems and servers to be inexpensive. Depending on the applications running on end systems, low cost is often more important than availability and performance in campus network designs. For enterprise networks, availability is usually more important than low cost. Nonetheless, customers are looking for ways to contain costs for enterprise networks. Recurring monthly charges for WAN circuits are the most expensive aspect of running a large network. To reduce the cost of operating a WAN, customers often have one or more of the following technical goals to achieve affordability:

- Use a routing protocol that minimizes WAN traffic.

- Consolidate parallel leased lines carrying voice and data into fewer WAN trunks.

- Select technologies that dynamically allocate WAN bandwidth—for example, ATM rather than time-division multiplexing (TDM).

- Improve efficiency on WAN circuits by using such features as compression.

- Eliminate underutilized trunks from the internetwork and save money by eliminating both circuit costs and trunk hardware.

- Use technologies that support oversubscription.

The second most expensive aspect of running a network, following the cost of WAN circuits, is the cost of hiring, training, and maintaining personnel to operate and manage the network. To reduce this aspect of operational costs, customers may require you to do the following as you develop the network design:

- Select internetworking equipment that is easy to configure, operate, maintain, and manage.

- Select a network design that is easy to understand and troubleshoot.

- Develop good network documentation that can help reduce troubleshooting time.

- Select network applications and protocols that are easy to use so that users can support themselves to some extent.

Making Network Design Tradeoffs

To meet high expectations for availability, redundant components are often necessary, which raises the cost of a network implementation. To meet rigorous performance requirements, high-cost circuits and equipment are required. To enforce strict security policies, expensive monitoring might be required and users must forgo some ease of use. To implement a scalable network, availability might suffer, because a scalable network is always in flux as new users and sites are added. Implementing good throughput for one application might cause delay problems for another application. Lack of qualified personnel might suggest the need for expensive training or the need to drop certain features. The network design that you develop must take these tradeoffs into consideration. One cause of network problems can be inadequate staffing and reduced training due to overzealous cost cutting. The tradeoff with cutting costs might be a network that isn’t robust or has substandard performance until the problem is recognized, which often takes a year or two. If the in-house network staff was cut, outsourcing might become a necessity, which could end up being more costly than it would have been to keep the in house staff.

The network design process is usually progressive. This means that legacy equipment must coexist with new equipment. Your design might not be as elegant as you would like because you might need for it to support old devices and old applications. If the new network is not being introduced at the same time as new applications, the design must provide compatibility with old applications. Also, be aware that insufficient bandwidth in parts of the network, where the bandwidth cannot be increased due to technical or business constraints, must be resolved by other means.

To help you analyze tradeoffs, ask your customer to identify a single driving network design goal. In addition, ask your customer to prioritize the rest of the goals. Prioritizing will help the customer get through the process of making tradeoffs.

One analogy that helps with prioritizing goals is the “kid in the candy store with a dollar bill” analogy. Using the dollar bill analogy, explain to the customers that they are like children in a candy store who have exactly one dollar to spend. The dollar can be spent on different types of candy: chocolates, licorice, jelly beans, and so on. But each time more money is spent on one type of candy, less money is available to spend on other types. Ask customers to add up how much they want to spend on scalability, availability, network performance, security, manageability, usability, adaptability, and affordability. For example, a customer could make the following selections:

| Scalability: | 20 |
| --- | --- |
| Availability: | 30 |
| Network performance: | 15 |
| Security: | 5 |
| Manageability: | 5 |
| Usability: | 5 |
| Adaptability: | 5 |
| Affordability: | 15 |

Total (must add up to 100): 100

Keep in mind that, sometimes, making tradeoffs is more complex than what has been described because goals can differ for various parts of an internetwork. One group of users might value availability more than affordability. Another group might deploy state of- the-art applications and value performance more than availability. In addition, sometimes a particular group’s goals are different from the overall goals for the internetwork as a whole. If this is the case, document individual group goals and goals for the network as a whole. Later, when selecting network technologies, you might see some opportunities to meet both types of goals—for example, choosing LAN technologies that meet individual group goals and WAN technologies that meet overall goals.

Technical Goals Checklist

You can use the following checklist to determine if you have addressed all your client’s technical objectives and concerns:

- I have documented the customer’s plans for expanding the number of sites, users, and servers for the next 1 year and the next 2 years.

- The customer has told me about any plans to migrate departmental servers to a centralized data center.

- The customer has told me about any plans to integrate data stored on a legacy mainframe with the enterprise network.

- The customer has told me about any plans to implement an extranet to communicate     with partners or other companies.

- I have documented a goal for network availability in percent uptime and/or MTBF and MTTR.

- I have documented any goals for maximum average network utilization.

- I have documented goals for network throughput.

- I have documented goals for pps throughput of internetworking devices.

- I have documented goals for accuracy and acceptable BERs.

- I have discussed with the customer the importance of using large frame sizes to maximize efficiency.

- I have discussed with the customer the tradeoffs associated with large frame sizes and serialization delay.

- I have identified any applications that have a more restrictive response-time requirement than the industry standard of less than 100ms.

- I have discussed network security risks and requirements with the customer.

- I have gathered manageability requirements, including goals for performance, fault, configuration, security, and accounting management.

❑ Working with my customer, I have developed a list of network design goals, including both business and technical goals. The list starts with one overall goal and includes the rest of the goals in priority order. Critical goals are marked as such.